SOP 1.0: Fingernail Isotopes

(Current and Updated Versions Located on the Archaeological Sciences

Laboratory Shared OneDrive) Last Revised: 17 February 2026

CONTENTS

Principle, Spirit, & Intent 									0.0

Purpose & Scope										1.0

Definitions										2.0

Background										3.0

Theoretical Framework									4.0

Materials & Equipment									5.0

Sample Cleaning Procedure									6.0

Drying Procedure										7.0

Subsampling and Encapsulation								8.0

Documentation 										9.0

Quality Assurance & Control									10.0

Safety Considerations									11.0

References Cited or Implied									12.0

Fingernail Sample Description and Acceptance Criteria     			                             	Appendix A

Reagents and Solvent Specifications							Appendix B

Equipment Specifications								Appendix C

Fingernail Sample Cleaning Checklist 							Appendix D

Subsampling and Encapsulation Guidelines      						Appendix E

Quality Assurance and Contamination Control 						Appendix F

Safety and Waste Disposal 								Appendix G

0.0 Principle, Spirit & Intent

This SOP establishes standardized procedures for cleaning, preparation, and subsampling of human fingernail clippings for isotopic and elemental analysis. The intent is to minimize contamination, ensure analytical reproducibility, and maintain consistency with published methodologies.

1.0 Purpose & Scope

This SOP describes the procedures used to clean, dry, weigh, and encapsulate fingernail clippings prior to laboratory analysis. It applies to all personnel preparing fingernail samples for stable isotope, trace element analysis, or related analytical techniques.

2.0 Definitions

Fingernail Clippings – Keratinized human nail material collected for biochemical or isotopic analysis

DI Water – Deionized water.

Ultrasonic Bath – Laboratory sonicator used to dislodge surface contaminants.

Tin Capsules – Pre-cleaned capsules used for combustion-based analyses.

3.0 Background

Human fingernails are composed primarily of keratin and can incorporate isotopic and elemental signatures reflective of diet, environment, and physiology. Because fingernails are exposed to external contaminants (e.g., dirt, oils, cosmetic residues), thorough cleaning is required prior to analysis.

The procedures described herein are adapted from methods outlines by O’Connell et al. and subsequent applications in isotope research to ensure removal of surface contaminants without altering endogenous chemical signals.

4.0 Theoretical Framework

Organic solvents such as chloroform and methanol are effective in removing lipids and exogenous organic contaminants from keratinous tissues. Ultrasonic agitation enhances the removal of residual particulates. Controlled drying ensures complete removal of solvents and moisture prior to weighing and encapsulation.

5.0 Materials & Equipment

- Deionized (DI) water

- Chloroform

- Methanol

- Glass vials with caps

- Ultrasonic bath

- Convection oven capable of 65 °C

- Analytical balance (±0.01 mg or better)

- Pre-cleaned tin capsules

- Forceps

- Nitrile gloves

- Fume hood

6.0 Sample Cleaning Procedure

### 6.1 Initial Cleaning

- Wearing nitrile gloves, inspect fingernail clippings for visible debris.

- Rinse samples thoroughly with DI water to remove surface dirt.

- Manually remove any remaining visible contaminants using clean forceps.

### 6.2 Organic Solvent Wash

- Place fingernail samples into clean glass vials.

- Add a 2:1 chloroform: methanol solution to fully submerge samples.

- Gently agitate then sit for 60 minutes in the 2:1 chloroform: methanol.

- Decant solvent and repeat wash for an additional 30 minutes.

- Conduct all solvent steps inside a certified chemical fume hood.

### 6.3 Ultrasonic Rinse

- Remove samples from solvents and rinse briefly with DI water.

- Place samples in fresh DI water and sonicate in an ultrasonic bath for 20–40 minutes.

- Replace DI water if visible residues are observed.

7.0 Drying Procedure

- Transfer cleaned samples to clean, labeled containers

- Dry samples in a convection over at 65C for 24-48 hours

- Ensure samples are completely dry prior to weighing.

8.0 Subsampling and Encapsulation

- Using clean forceps or scissors, cut fingernail samples into portions weighing 500 µg ±50 µg.

- Verify mass using an analytical balance.

- Place each subsample into a pre-cleaned tin capsule

- Seal capsules securely to prevent material loss

- Label capsules with sample ID corresponding to laboratory records

9.0 Documentation

For each sample, record the following:

- Sample ID

- Cleaning date

- Operator initials

- Drying duration

- Final sample weight

- Capsule ID

All records must be retained in the project laboratory notebook or digital database.

10.0 Quality Assurance & Control

- Solvents must be fresh and uncontaminated.

- Tin capsules must be pre-cleaned and inspected prior to use.

- Analytical balance calibration must be verified daily.

- Replicate subsamples should be prepared when sample size permits.

11.0 Safety Considerations

- Chloroform and methanol are hazardous chemicals; handle only in a fume hood.

- Wear appropriate PPE: Lab coat, gloves, and eye protection.

- Dispose of solvent waste according to institutional hazardous waste protocols.

- Use caution when operating ultrasonic baths and convection ovens.

12.0 References Cited or Implied

Mancuso, C. J., & Ehleringer, J. R. (2019). Resident and nonresident fingernail isotopes reveal diet and travel

patterns. Journal of Forensic Sciences, 64(1), 77-87.

O’Connell TC, Hedges REM, Healey MA, Simpson AHRW. Isotopic comparison of hair, nail and bone: modern

analyses. J Archaeol Sci 2001;28(11):1247–55

## Appendix A: Fingernail Sample Description and Acceptance Criteria

## Referenced in: Sections 1.0 (Purpose & Scope), 6.0 (Sample Cleaning)

### A.1 Sample Description

Fingernail samples consist of keratinized human nail clippings collected for isotopic and/or elemental analysis. Samples may originate from archaeological, historical, forensic, or modern comparative contexts, depending on project scope and approval.

### A.2 Acceptance Criteria

Samples are acceptable for analysis if they:

- Are free of visible soil, polish, adhesives, or cosmetic residues

- Have sufficient mass to permit subsampling (≥1 mg total preferred)

- Are accompanied by appropriate documentation and chain-of-custody records

Samples may be rejected if:

- They contain visible foreign material after initial cleaning

- They show evidence of chemical treatment (e.g., nail polish remover residue)

- Total mass is insufficient for analytical requirements

## Appendix B: Reagents and Solvent Specifications

Referenced in: Section 6.0 (Sample Cleaning Procedure)

### B.1 Approved Reagents

- Deionized water (≥18 cm)

- Chloroform (HPLC or analytical grade)

- Methanol (HPLC or analytical grade)

### B.2 Solvent Preparation

- Prepare chloroform: methanol solution at a 2:1 volumetric ratio.

- Solutions must be freshly prepared or verified as uncontaminated prior to use.

- Label all solvent containers with contents, date, and preparer initials.

## Appendix C: Equipment Specifications

Referenced in: Sections 5.0, 7.0, and 8.0

### C.1 Ultrasonic Bath

- Capable of continuous operation for at least 40 minutes

- Must be cleaned and filled with fresh DI water prior to use

### C.2 Convection Oven

- Capable of maintaining 65 °C ± 2 °C

- Temperature must be verified annually or per institutional policy

### C.3 Analytical Balance

- Precision of ±0.01 mg or better

- Calibration verification required prior to weighing sessions

## Appendix D: Fingernail Sample Cleaning Checklist

Referenced in: Section 6.0

☐ Sample ID verified
☐ Visible debris removed with DI water
☐ First solvent wash completed (60 min)
☐ Second solvent wash completed (30 min)
☐ Ultrasonic DI water rinse completed (20–40 min)
☐ Sample transferred to drying container
☐ Operator initials and date recorded

## Appendix E: Subsampling and Encapsulation Guidelines

Referenced in: Section 8.0

### D.1 Subsampling

- Target subsample mass: 500 µg ±10%

- Use clean forceps or scissors between samples to prevent cross-contamination.

- If sample mass permits, prepare replicate subsamples.

### D.2 Tin Capsule Handling

- Capsules must be pre-cleaned and free of oils or residues.

- Capsules should be folded tightly to prevent sample loss.

- Capsules must be labeled or traceable to sample ID via documentation.

## Appendix F: Quality Assurance and Contamination Control

Referenced in: Section 10.0

- Perform blank capsule checks periodically.

- Use solvent-only controls when processing small sample batches.

- Maintain separation between cleaned and uncleaned samples at all times.

- Document any deviations from the SOP in analytical notes.

## Appendix G: Safety and Waste Disposal

Referenced in: Section 11.0

- Collect chloroform: methanol waste in approved hazardous waste containers.

- Label waste containers according to institutional policy.

- Dispose of solvent waste through Environmental Health & Safety (EHS).

- Follow institutional procedures for ultrasonic bath and oven operation.